PWCM / FUNCTION DEFINITION MATRIX / REV B / SHEET 1 OF 2

Front Door Power Window Control Module — condensed engineering sheet

A. MODE / CONDITION MATRIX

| Ref | Mode | Trigger | Ends on | Express? | Lockout affected? | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| A-01 | Manual up | SW_UP_D1 held | Switch release / end stop | No | Yes (non-driver) | Continues only while held. Occupant is deemed to be supervising the movement. |
| A-02 | Manual down | SW_DN_D1 held | Switch release / end stop | No | Yes (non-driver) | As A-01, opening direction. |
| A-03 | Express up | SW_UP_D2 | End stop / obstruction / interrupt | Yes | Yes (non-driver) | Driver door only. Requires valid normalisation record. Unsupervised — obstruction detection mandatory. |
| A-04 | Express down | SW_DN_D2 | End stop / interrupt | Yes | Yes (non-driver) | Both front doors. |
| A-05 | Remote close | CAN_WIN_REQ | End stop / obstruction | Yes | No | Issued by BDC. Treated as unsupervised. |
| A-06 | Reversal | Obstruction detected | ≥300 mm travelled / fully open | n/a | No | Not interruptible. Overrides any pending request. |

B. PARAMETER SET (READ ACROSS BOTH HALVES)

| ID | Parameter | Value | Note | ID | Parameter | Value | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P-01 | Max squeezing force | 100 N | Detection zone only | P-07 | Travel time express up | 1.7 s | ±0.2 s @13.5V/23°C |
| P-02 | Detection zone lower | 4 mm | From mating structure | P-08 | Operating voltage | 9.0–16.0 V | Full function |
| P-03 | Detection zone upper | 200 mm | From mating structure | P-09 | Degraded voltage | 6.5–9.0 V | Manual only |
| P-04 | Halt time on detect | 100 ms | Maximum | P-10 | Operating temp | −40…+85 °C | Substrate |
| P-05 | Reversal travel | ≥300 mm | Or fully open | P-11 | Cycles before derate | 12 | Consecutive full travel |
| P-06 | Travel time express down | 1.4 s | ±0.2 s @13.5V/23°C | P-12 | Switch service after IGN off | TBD | Target 10 min |

C. SIGNAL LIST

| Signal | Dir | Type | Description | Signal | Dir | Type | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SW_UP_D1 | In | Discrete | Driver up, detent 1 | MTR_PWM | Out | PWM | H-bridge duty command |
| SW_UP_D2 | In | Discrete | Driver up, detent 2 | MTR_DIR | Out | Discrete | Direction select |
| SW_DN_D1 | In | Discrete | Driver down, detent 1 | CAN_WIN_STAT | Out | Network | Position, state, availability |
| SW_DN_D2 | In | Discrete | Driver down, detent 2 | CAN_WIN_DTC | Out | Network | Active DTCs |
| SW_LOCKOUT | In | Discrete | Rear lockout state | — | — | — | — |
| HALL_A / HALL_B | In | Pulse | Quadrature; position + load | — | — | — | — |
| V_BATT | In | Analogue | Door harness supply | — | — | — | — |
| T_MOD | In | Analogue | Substrate temperature | — | — | — | — |
| CAN_WIN_REQ | In | Network | BDC window command | — | — | — | — |

D. BEHAVIOURAL NOTES

| Ref | Description |
| --- | --- |
| D-01 | Express up sequence. On command the module verifies a valid normalisation record and that no obstruction fault is latched, then energises the motor in the closing direction while monitoring load continuously. Approaching the upper seal the module enters the seal engagement region where the load rise caused by the seal is expected and is not treated as an obstruction. On observing the seated load signature the module de-energises and records the closed end stop. If that signature is not observed within the expected travel the module de-energises, discards the normalisation record and raises a position plausibility fault; express up is then unavailable until a manual close has been held to the end stop. |
| D-02 | Detection suppression. Within 4 mm of the mating structure the reversal function is suppressed. The residual gap is too small to admit a body part, and suppression avoids nuisance reversals from seal friction, ice or minor misalignment. |
| D-03 | Position retention. The module retains learned end-stop positions and last known glass position across ignition cycles, so a routine restart does not require the occupant to re-normalise before express movements are again available. |
| D-04 | Responsiveness. The window shall respond quickly to switch actuation so that the occupant does not perceive a delay between operating the switch and the glass beginning to move. |
| D-05 | Repeated obstruction. Three obstruction events within one closing attempt sequence inhibit further automatic closing until ignition is cycled. Manual supervised closing remains available. |
| D-06 | Communication loss. With the BDC unreachable the module continues to service local switch inputs using the last known lockout state. Vehicle-level commands are not actioned. On restoration the module resynchronises reported status before accepting new vehicle-level commands. |

Regulatory basis: publicly available regulation in the United States and under the UNECE framework requires a power-operated window closing without continuous occupant supervision to reverse before exerting a squeezing force of 100 N on an obstruction located between 4 mm and 200 mm from the structure against which the window closes. P-01 to P-03 derive from this and are not re-calibratable without formal change.